系统性失效措施

| 拟制 |  |
| --- | --- |
| 审核 |  |
| 批准 |  |
| 日期 |  |

修订历史

| 版本 | 日期 | 作者 | 审核 | 文档状态 | 修改内容 |
| --- | --- | --- | --- | --- | --- |

注：文档状态：“草稿“，”已审核“，”已发布“

Contents

1	目的	3

2	参考文档	3

3	系统性失效措施	3

目的

本文档描述在产品开发过程中使用的措施以防止系统性失效，用于证明系统性失效被有效的避免。

参考文档

| 文档名称 | 版本 |
| --- | --- |
| IEC 61508 | 2010 |
| ISO 13849-1:2023 | 2023 |

系统性失效措施

Measures for the control of systematic failures

| - Measures for the control of systematic failures |  |
| --- | --- |
| - Use of de-energization: The SRP/CS should be designed so that the machine will achieve or maintain a safe state upon a power supply loss. | - YES/NO - 选用了就是YES，没选用就是NO |
| - Measures for controlling the effects of voltage breakdown, voltage variations, overvoltage, - undervoltage: SRP/CS behaviour in response to voltage breakdown, voltage variations, overvoltage, - and undervoltage conditions should be predetermined so that the SRP/CS can achieve or maintain - a safe state of the machine |  |
| - Measures for controlling or avoiding the effects of the physical environment (e.g. temperature, - humidity, water, vibration, dust, corrosive substances, EMI and its effects): SRP/CS behaviour in - response to the effects of the physical environment should be predetermined so that the SRP/CS - can achieve or maintain a safe state of the machine Measures for controlling or avoiding the effects of the physical environment (e.g. temperature, humidity, water, vibration, dust, corrosive substances, EMI and its effects): SRP/CS behaviour in response to the effects of the physical environment should be predetermined so that the SRP/CS can achieve or maintain a safe state of the machine |  |
| - Program sequence monitoring should be used with SRP/CS containing software in order to detect - defective program sequences: A defective program sequence exists if the individual elements of a - program (e.g. software modules, subprograms or commands) are processed in the wrong sequence - or period of time or if the clock of the processor is faulty |  |
| - Measures for controlling the effects of errors and other effects arising from any data communication process |  |
| - failure detection by automatic tests; |  |
| - tests by redundant hardware |  |
| - diverse hardware |  |
| - operation in the positive mode |  |
| - mechanically linked contacts |  |
| - direct opening action |  |
| - oriented mode of failure |  |
| - over-dimensioning by a suitable factor, where the manufacturer can demonstrate that over dimensioning - improves reliability |  |

Measures for avoidance of systematic failures during SRP/CS design

| - Measures for avoidance of systematic failures during SRP/CS design |  |
| --- | --- |
| - Use of suitable materials and adequate manufacturing：Selection of material, manufacturing methods and treatment in relation to, e.g. stress, durability, - elasticity, friction, wear, corrosion, temperature, conductivity, dielectric rigidity. | - YES/NO - 选用了就是YES，没选用就是NO |
| - Correct dimensioning and shaping：Consideration of, e.g. stress, strain, fatigue, temperature, surface roughness, tolerances, manufacturing. |  |
| - Proper selection, combination, arrangements, assembly and installation of components, including - cabling, wiring and any interconnections; Apply appropriate standards and manufacturer’s application notes, e.g. catalogue sheets, installation instructions, specifications, and use of good engineering practice. |  |
| - Compatibility; Use components with compatible operating characteristics |  |
| - Withstanding specified environmental conditions; Design the SRP/CS so that it is capable of working in all expected environments and in any foreseeable adverse conditions, e.g. temperature, humidity, vibration and EMI |  |
| - Use of components designed to an appropriate standard and having well-defined failure modes. - To reduce the risk of undetected faults by the use of components with specific characteristics |  |
| - Hardware design review (e.g. by inspection or walk-through); To reveal by reviews and analysis discrepancies between the specification and implementation. |  |
| - Computer-aided design tools capable of simulation or analysis; Perform the design procedure systematically and include appropriate automatic construction elements that are already available and tested. |  |
| - Simulation. Perform a systematic and complete inspection of an SRP/CS design in terms of both the functional - performance and the correct dimensioning of their components. |  |

Measures for avoidance of systematic failures during SRP/CS integration

| - Measures for avoidance of systematic failures during SRP/CS design |  |
| --- | --- |
| - functional testing | - YES/NO - 选用了就是YES，没选用就是NO |
| - project management |  |
| - documentation |  |